| Author(s) and year | Year | Type of solar farm | Main objective | Method(s) | Environmental factors | Economic factors | Social factors | Technical factors | Key findings | Future recommendations |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (Sicuaio, Zhao et al. 2024) |  | Solar PV farms | Selecting suitable sites for solar farms in Mozambique by optimizing multiple conflicting economic, environmental, and technical criteria. | Multi-objective optimization (MOO), QGIS | Solar radiation, minimizing slope, maximizing aspect, Land use and land cover | Regulatory restriction | Close to urban area | Connection to electricity grid, road network | Article identifies optimal locations for solar farms in Mozambique. Highlights the importance of considering diverse stakeholder preferences in the decision-making process and emphasizes that strategic site selection can contribute to achieving multiple SDGs. | More socio-economic factors can be included |
| (Vagiona, Tzekakis et al. 2022) |  | Offshore Solar Farm (OSF) | Identify and rank suitable OSF sites in the Aegean Sea, Greece | GIS, MCDM, TOPSIS | Restriction zones, water depth, distance from shore, distance from ports, solar radiation | Installation site area | Serving population |  | Solar radiation is the most influential assessment criterion (37.97%) | Lack of relevant national legislation concerning OSF deployment, technical factors, |
| (Saraji, Streimikiene et al. 2024) |  | Solar photovoltaic constructions | Identify the most suitable locations among 39 potential cities in the Baltic region for constructing solar PV farms | GIS, DEA, CRITIC_TOPSIS, | Solar radiation, air temperature,slope | Proximity to urban areas, roads, power lines, GDP | Land acquisition | Irradiation, elevation, sunashine duration | The findings align with the Global Solar Atlas when excluding social and economic aspects, highlighting the importance of both technical feasibility and broader decision-making criteria for optimal solar farm site selection. | Apply the two-stage model in other countries to evaluate its applicability |
| (Chai, Kong et al. 2024) |  | PV farms | Determine suitable areas for solar PV farms | MCDM, QGIS | Slope aspect, | Proximity to ecological sources, ecological corridor | Policy constraints | Wind speed | Indetified 5563.02 km² of potential PV deployment areas in Qilian and proposed three “PV+” models to promote sustainable solar energy development in ecologically fragile regions | Using technical factors for further research |
| (Li, Tan et al. 2024) |  | Photovoltaic farms | Optimize solar farm site selection in desert environments by assessing the impact of sand flux, sand burial, and dust contamination | Custom scoring scheme based on flux potential (FP) and resultant flux potential (RFP) | Wind speed, elevation, proximity to protected areas |  |  | Sand flux, sand burial, dust contamination, land wind | 47.2% of existing desert solar installations are in high-scoring regions, provides a guide for selecting optimal sites and contributing to global energy sustainability goals | Implementation of economic and social factors in further research |
| (Bandira, Tan et al. 2022) |  | Solar farms | The study aims to identify optimal solar farm locations in the George Town Conurbation | GIS-based MCDM | Climatic criteria, orography, environmental | Economic criteria | Restricted areas |  | Identifies suitable solar farm locations in the George Town Conurbation, with validation showing no significant difference between NASA POWER and ground-measured data, demonstrating its potential for solar site selection in Malaysia | Limitations in local-scale resolution and climate data availability |
| (Nguyen, Truong et al. 2022) |  | Solar panels | Aims to develop a GIS-based simulation for choosing the best location for a solar farm in Binh Thuan province in South-Central Vietnam | GIS-based simulation | Solar radiation, climate, location, orography | Land use |  | Distance from roads, residental areas | Effective solar farm planning requires GIS-based models, infrastructure improvements, and strategies for transmission challenges and panel disposal. | No social factors considered |
| (Chen, Huang et al. 2014) |  | Solar farms | Establish a decision model for improving the performance of solar farms. | MCDM, GIS, DEMATEL technique | Agrological capacity, orography, climatology |  |  | Distance to roads, power lines, villages, substations | Solar radiation is the most important criterion impacting solar farms site selection, followed by average temperature and distance to villages. | No social and economic factors considered |
| (Dhunny, Doorga et al. 2019) |  | Solar, wind-solar farms | Develops analytical framework using fuzzy logic to evaluate optimal sites for wind, solar and hybrid wind-solar farms | GIS, Fuzzy logic modelling | Climatological, topographic factors |  | Human factors, Recreational areas, agricultural land |  | Fuzzy-logic modeling effectively identifies optimal hybrid wind-solar farm locations, offering a flexible tool for energy planners in complex terrains | Lacks economic and technical factors |
| (Amjad and Shah 2020) |  | Solar farms | Prioritizes areas with maximum solar energy potential to identify and group high-potential sites. | QGIS | Geography, solar radiation |  |  | Zone evaluation, size, location | Identifies high-potential solar farm sites, ensuring optimal solar irradiance and minimal terrain ruggedness. | Excludes economic and infrastructure factors |
| (Onuoha, Denwigwe et al. 2025) |  | Photovoltaic farms | Aims to identify optimal sites for utility-scale solar PV farms in Ikorodu, Lagos State, Nigeria | GIS, MCDM | Temperature, ecological restrictions | Cost minimization | Proximity to settlements, cultural sites | Solar irradiation, proximity to grid infrastructure | In Nigeria, 68.77% of the land is unsuitable for development, with only 17.78% classified as highly suitable and 12.67% as moderately suitable. Marginally suitable and most appropriate areas are minimal, at 0.73% and 0.04% | Including additional factors such as changes in land use, community perceptions. |
| (Kırcalı and Selim 2021) |  | Solar farms | Developing suitability map for solar farms | GIS, MCDM | Solar parameters, Geography, slope, aspect, elevation, land use |  |  | Logistics: distance to transformers, roads, residential areas | Despite having high potential areas very little of the suitable area is used for solar energy generation | Lacks social and economic factors |
| (Demir, Dinçer et al. 2023) |  | PV farms | Find the suitable locations for large-scale PV farms | GIS, optimality-based site growing (OBSG) | Solar radiation, | Government support | Renewable energy market | Proximity to major cities, infrastructure | The novel method effectively determines the most suitable locations for large-scale PV plants | Excluding different capacity farms in ranking |
| (Asadi and Pourhossein 2021) |  | Wind/solar farms | Aims to to enhance site selection for wind/solar power plant. | GIS, MCDM | Global irradiance, wind |  |  | Land slope, air pressure, distance | Identifies classes for wind/solar site construction | Lacks social, economic factors |
| (Ali, Taweekun et al. 2019) |  | Wind and solar farms | To explore Songkhla Province, Thailand for siting wind and solar farms | GIS, MCDM | Climate, topography, land use | Economic viability |  |  | Study finds that Ranot District has the highest suitability for both wind and solar utility-scale farms in Songkhla Province. | Lacks social and technical factors |
| (Barzehkar, Parnell et al. 2021) |  | Photovoltaic solar power plants | Aims to optimize wind and solar power plant locations | AHP, GIS, fuzzy logic, and WLC | Solar radiation, wind speed, slope | Distance from roads, transmission lines, land use | Distance from population centers |  | 26% of Isfahan Province is highly suitable for solar farms, while 18% is highly suitable for wind farms | Lacks technical factors |
| (Günen 2021) |  | PV farms | Aims to find optimal sites for solar PV farms | GIS, MCDM, AHP | Climate, geography |  | Location, railway, road, powerline |  | Optimal sites for solar PV farms in Kahramanmaraş, Turkey are mapped | Lacks economic and technical factors |
| (Rane, Günen et al. 2024) |  | Photovoltaic power plants | Identifying optimal locations for solar PV farms by integrating various spatial factors | GIS, MIF | Radiation, humidity, wind speed, aspect | Land use | Distance from roads | Land surface temperature | 17.96% of the study area is highly suitable for solar PV farms, with an overall prediction accuracy of 81.80% | Using GIS-MIF method for other areas |
| (Kuru 2023) |  | Solar power plants | Identify optimal solar power plant locations by balancing economic efficiency and ecological sensitivity | GIS, MCDM, AHP | Global horizontal irradiation, solar radiation, sunshine duration, aspect, topographic shadow | Average land cost |  | Distance to conservation areas, ecosystem services, land cover, natural integration potential, vegetation index, land use capability | 76% of the study area requires ecological protection, while only a negligible portion is highly suitable for solar power plants, emphasizing the challenge of balancing economic and environmental considerations | Lacks social factors |
| (Robles-Algarín, Castrillo-Fernández et al. 2024) |  | Photovoltaic solar farms | Optimal site selection of photovoltaic solar farms | CRITIC, TOPSIS, PCA | Meteorological criteria, Environmental Criteria | Financial Criteria | Demographic Criteria | System Performance Criteria | Barranquilla consistently ranked among the top two most suitable locations for solar farms, while Sincelejo ranked among the lowest, with the choice of weighting method impacting the final rankings | Utilizing multi-criteria evaluations in various contexts |
| (Wang, Chung et al. 2023) |  | Solar power plants | To optimize solar power plant site selection in Indonesia | DEA, F-AHP, F-MARCOS | Climate, conserving energy, endangered species | Electricity consumption, cost, proximity to residential areas | Residents attitude, government rules and regulations | Geology, availability of skilled workers | Identified optimal solar power plant locations in Indonesia, contributing to cost-effective and sustainable renewable energy development | Exploration of the potential of renewable energy sources in Indonesia and other countries |
| (Li, Dong et al. 2024) |  | PV power plants | To optimize site selection for large-scale PV power plants, addressing land consolidation and economic feasibility | DBSCAN clustering, GIS | Annual solar radiation, average temperature, slope, aspect | Land use |  | Electricity network, transport network | Identified 425,191 km² of developable land for PV installation in China, but only 23% consisted of consolidated land parcels suitable for large-scale PV development, with the potential to supply 90% of China's electricity demand | Identifying agricultural lands where PV facilities can be developed. |
| (Mehrian, Qelichi et al. 2024) |  | Solar power plants | Evaluates the relative potential of different regions in Iran for solar power plant development | GIS, FIS | Protected land, wildlife areas, topography | Distance to power lines, urban areas, roads |  | Solar irradiation, average temperature, annual dusty days | Identified Isfahan, Khorasan-Razavi, and Kerman provinces as the most suitable for PV power plant construction, with over 70% of the highly suitable land in these regions | Further exploration of socio-economic factors |
| (Yılmaz, Dinçer et al. 2023) |  | PV farms | Explores the suitability of Menteşe Region in Türkiye for the installation of solar PV farms | GIS, AHP | Orography, flood hazard index, erosion hazard index, | Land use, distance from transmission lines, populated areas |  |  | Study reveals that flood hazard and erosion indices affect the suitable sites | Lacks technical and social factors |
| (Vrînceanu, Dumitrașcu et al. 2022) |  | Photovoltaic farms | Aims to analyze the site suitability for photovoltaic farms in Romania | GIS, MCE | Global horizontal radiation, altitude, slope, multiannual average sunshine duration, temperature, wind speed | Land use, distance to settlements, protected areas |  |  | The suitability map reveals that over 29.9% of Romania’s area has high potential for photovoltaic farm location and expansion | Exploration of social and technical factors |
| (Noorollahi, Senani et al. 2022) |  | PV solar farm | Aims to locate and evaluate a photovoltaic solar power plant's potential in Khuzestan province | Fuzzy boolean logic, GIS, AHP | Climate, orography, | Land use, proximity to roads, transmission line | Proximity to urban areas, villages |  | 25.78% of the province’s area has the best suitability for the solar farm | Lacks technical factors |
| (Hasti, Mamkhezri et al. 2023) |  | Solar photovoltaic farm | Optimal solar photovoltaic site selection for Kurdistan province, Iran | MCDM-AHP, TOPSIS, GIS | Meteorological, | Land use | Location of dams, lakes, rivers, protected areas | Human infrasturture | Construction of solar farms yields substantial positive economic and environmental impacts for the region | Calculation and prediction of the volume of local and seasonal micro-dust |
| (Asadi, Pourhossein et al. 2023) |  | Hybrid Solar and Wind Power Plant | Selecting the optimal locations for wind and solar farms by mapping the space of the decision criteria to the site score | AHP, GIS | Distance from urban areas and distance from protected areas | Land slope, distance from power lines, and distance from road networks |  | Wind-power density (WPD) and solar irradiation | Areas with an area of about 9, 4 and 7 km2 are suitable for the construction of wind, solar and wind/solar power plants | Further research including power and cost factors |
| (Raza, Yousif et al. 2023) |  | Solar PV farms | Site suitability for utility scale solar PV and wind energy projects in Pakistan | GIS, MCDM, AHP | Climate, orography | Location |  | Wind Power Density | 25.28% area suitable for utility scale solar PV farms | Lacks social factors |
| (Hasanzaeh, Kamran et al. 2024) |  | Solar power plants | Finding the optimal location for a solar power plant | MCDA, GIS, AHP | GHI, Temperature, Elevation, Slope | Land cover |  | Distance from city, and Distance from road | Best regions have been identified for higher potential for solar energy generation | Lacks social factors |
| (Hooshangi, Gharakhanlou et al. 2023) |  | Photovoltaic power plants | Prioritizing potential areas to localize solar farms in Iran | GIS-based FF-TOPSIS, FFs | Climate, geography | Land cost, Energy consumption, Land use | Population, distance from population centres | Distance from main roads, transmission lines, synoptic stations | Approximately 13.81% of the entire country has the potential for solar power plants | Site selection for provincial scales and neighboring provinces. |
| (Vagiona 2021) |  | PV farms | To evaluate and rank suitable locations for solar photovoltaic farms in Rhodes, Greece | MCDM, TOPSIS, AHP, GIS | Altitude | Land cover |  | Installation site area limitations, distance from protected areas, airports, | Highlights the importance of using multiple MCDM techniques for robust decision-making in solar farm site selection | Lacks social factors |
| (Sánchez-Lozano, Antunes et al. 2014) |  | Solar farms | Evaluation and classification of best sites to implant photovoltaic solar farms | MCDM, ELECTRE-TRI, GIS | Climate, orography, climate |  |  | Location, distances | Structured evaluation of land suitability for solar farm installation | Lacks social and economic factors |
| (Mokarram, Mokarram et al. 2020) |  | Solar farms | Determine the optimal location for solar farm development | GIS-MCDA, Fuzzy AHP, Fuzzy ANP | Elevation, slope, climate | Land use | Distance to residential areas | Distance to roads, | Decreasing specific parameters, such as DPTL and distance to roads, improves site suitability. The northern region of the study area is identified as the most suitable location for solar farms | The spatial placement of solar farms can be combined with the problem of power network management |
| (Hosouli and Hassani 2024) |  | Solar plants | Solar plant location selection in UK | MCDM | Temperature, annual sunshine hours, humidity |  |  | Distance from major road, power network, potential demand | Torquay was identified as the best site for a solar farm in the UK | Lacks socio-economic factors |
| (Demir, Dinçer et al. 2024) |  | PV farms | Selection of suitable sites for solar photovoltaic (PV) farms in Konya, Turkey | AHP, GIS | Solar radiation, slope | Land use, restricted areas | Distance to residential areas | Distance from transmission lines, road | The weight adjustments result in suitability area changes ranging from 3.4% to 50%, depending on the difference between conventional AHP weights and the predefined objective relations. This approach enhances the objectivity, accuracy, and reliability of site selection, particularly in regions where criteria relationships can be objectively defined. |  |
| (Doorga, Rughooputh et al. 2019) |  | Solar farms | To identify optimum solar farm sites in Mauritius | AHP, MCDM, GIS | Climatography, orography, topography |  | Exclusion areas, airports, tourist sites, etc. |  | Best sites identified for solar farms are located in the northern region of Mauritius | Lacks economic and technical factors |
| (Uyan and Dogmus 2023) |  | Solar farms | Selecting the most suitable locations for solar power plants in the Cumra Region, Konya, Turkey | ANP, GIS | Aspect |  | Distance from residential areas | Distance from transmission lines, surface waters, transformer centre, roads and railways | Highlights the importance of a structured, data-driven approach to solar farm placement to enhance energy efficiency and sustainability | Lacks economic factors |
| (Nafi'Shehab, Faisal et al. 2024) |  | Solar farms | To evaluate and assess the suitable locations for installing solar farms in Erbil governorate, Iraq | GIS, MCDM | Radiation, slope, aspect, elevation | Land use | Distance from urban areas | Distance to power lines, roads | The southeastern region of Erbil was identified as the most suitable area for solar farm construction due to favorable conditions and expansion potential |  |
| (Rogna 2020) |  | Solar farms | Selecting optimal locations for large-scale solar plants | MCDM, GIS |  | Plant cost, interest rate, operation costs |  | Land size, PV peak power, plant lifetime | Sicily and Sardinia are the Italian regions with the highest number of optimal and economic viable locations | The screening of more restricted zone, where to place real plants |
| (Sward, Nilson et al. 2021) |  | Photovoltaic farms | Aims to improve the GIS-MCDA approach for solar farm site selection by incorporating social considerations alongside technical and economic factors | GIS, MCDA | Solar resource, slope, elevation, climate | Land cover, cost of electricity, protected areas, government policy | Developed areas, public opinion/acceptance | Distance to electric infrastructure, distance from roads | Public opposition often stems from inadequate consideration of community interests and perceived fairness in energy siting | Further consideration of social criteria – aspects of the people and places |
| (Suh and Brownson 2016) |  | Solar farms | Evaluate the suitability of solar farm locations on Ulleung Island, Korea | GIS_MCE | Solar irradiation, sunshine hours, temperature in summer, slope |  |  | 3D path distance form nearest transmission line, nearest road | Identified three highly suitable regions for solar farm development | Lacks economic and social factors |
| (Finn and McKenzie 2020) |  | Solar farms | Suitability for solar farms was assessed for Northern Ireland | MCDM, AHP, GIS | Solar, orography, radiation | Constraint, land use |  | Road proximity, location | 462 potential sites were identified across the study area | Lacks social factors |
| (Prieto-Amparán, Pinedo-Alvarez et al. 2021) |  | Solar farms | Aims to evaluate site suitability for solar farm development in the Desert of Chihuahua, Mexico | RM, AHP, GIS | Solar radiation, temperature, slope, aspect, wind speed, landforms | Land use |  | Distance to main roads, vapor pressure, soil texture | Identified a larger suitable area (4983 km²) compared to the more stringent Ranking Method (RM) (1237 km²), with the northern region being the most suitable for solar farm development | Lacks socio-economic factors |
| (Ünsal, Demir et al. 2024) |  | Solar power plant | Aims to identify the most suitable locations for solar power plant construction in Mersin province | F-LMAW, GIS | Slope, elevation, aspect, GHI, temperature, cloudiness, number of snowy days | Land use capability, planning |  | Distance from the road, water resource, disaster center, power lines, transformer center, soil depth | Silifke and Mut regions are the most suitable locations for solar power plants, with solar radiation being the most influential criterion and distance from rivers the least influential | Lacks social factors |
| (Ara, Zohora et al. 2024) |  | PV farms | To determine the optimal location for solar plant establishment in Bangladesh | AHP | Sunshine radiation, sunshine duration |  | Proximity to residential area | Distance to transmission lines, road, natural disaster | Rajshahi as the most suitable location for a solar plant, followed by Ishwardi, Chattogram, Chuadanga, and Mymensingh. | Lacks socio-economical factors |
| (Khan, Ali et al. 2023) |  | PV power plants | To identify suitable locations PV power plants by analyzing geographic, technical, economic, and flood susceptibility factors to maximize efficiency and minimize costs | FUCOM, GIS | Geographical, flood susceptibility, | Distance to populate areas, roads, transmission lines |  | Solar irradiance, average temperature | 2.02% of the country's area is suitable for PV power plants, categorized into five suitability classes, highlighting the distribution of optimal solar plant locations | Lacks social factors |
| (Yilmaz, Kocer et al. 2024) |  | Solar power plants | To determine the most suitable locations for solar power plants in Antalya, Burdur and Isparta | FAHP, GIS | Climate, topography, land use | Distance to main roads, settlment areas, transmission lines |  |  | Approximately 20% of the Western Mediterranean Region (WMR) is suitable for SPP installation | Lacks economic and technical factors |
| (Ayough, Boshruei et al. 2022) |  | Solar power plants | To propose a new approach for solar farm site selection | ISAW, PROMETHEE | Land use, agrological capacity, carbon emission savings, distance from water bodies | Distance to transmission lines, maintenance cost, distance from urban areas, initial investment | Public acceptance, impact on local economy, employment and tourism | Radiation, slope, water bodies, roads | Refines the ranking of alternatives iteratively until the decision-maker is satisfied, offering a transparent and adaptable decision-making process for site selection | Further studies could improve the results by considering more criteria that affect the site selection process |
| (Tavana, Arteaga et al. 2017) |  | Solar farms | To introduce a three-stage fuzzy evaluation framework designed to locate solar farms | ANFIS, AHP | Distance from environmental areas, access to land, solar radiation | Distance from roads, distance from transmission power lines |  |  | The proposed framework successfully identifies the most suitable locations for solar power farms in the Iranian regions of Kerman and Yazd | Lacks technical, socio-economical factors |
| (Wati and Meukam 2024) |  | PV facilities | To assess the impact of climate change on the suitability of recommended locations for solar PV facilities in Cameroon | AHP, LSI, GIS | Climate, orography, vegetation |  |  |  | Findings indicate that 13.7% of the study area is suitable for utility-scale PV power plants, mainly in the Extreme-North region | Lacks, socio-economic and technical factors |
| (Pojadas and Abundo 2022) |  | PV and wind farms | Selecting sites for Variable Renewable Energy (VRE) farms | GIS, CBA | Annual carbon savings | Total costs, benefits, total NPV, | Local jobs |  | The framework was applied to assess the viability of utility-scale solar PV and wind farms in Bohol, revealing a modest potential for viable VRE projects while highlighting the influence of compensation effects in linear combination models | The method is applied only for cost efficiency |
| (Zhang and Zhao 2025) |  | SPV farms | Selecting optimal solar photovoltaic (SPV) sites by emphasizing the importance of matching resource availability with energy demand | MCDM, GIS | Resource side, | Demand side, |  |  | The proposed method was applied to SPV site selection in Beijing, identifying Shunyi as the priority location | Lacks social and technical factors |
| (Guaita-Pradas, Marques-Perez et al. 2019) |  | Grid-connected photovoltaic (GCPV) systems | comprehensive approach to selecting optimal sites for large grid-connected photovoltaic systems by integrating legal, political, environmental, and technical-economic criteria | GIS | Climate, internal soil, external soil | Design, initial capital cost, discount rates, lifetime of projects, retail price | Landscape impact, job opportunities, rural development, legal restrictions |  | Identifies suitable locations for solar farm development in the Valencian Community, Spain | Doesn’t incorporate technical fators |
| (Wang, Dang et al. 2021) |  | PV plant | Develops a two-stage approach to identify optimal locations for solar photovoltaic (PV) farms in Taiwan | DEA, AHP | Topography, wildlife habitat, human safety, land availability | Electricity consumption demand, construction cost, maintenance cost, transmission cost | Life quality of residents, support mechanism, social regulatory compliance, government policies and laws | Distance from network, urban area, industrial park, infrastructure | The analysis ranks Tainan, Changhua, and Kaohsiung as the top three most suitable cities for solar PV development | Hybrid renewable energy systems such as solar PV-wind and solar-biomass should be considered for Taiwan to obtain more cost-effective and technically feasible renewable energy source projects |
| (Hassan, Alhamrouni et al. 2023) |  | PV plants | Aims to assess site suitability and technical potential for large-scale solar PV systems | MCDM, CRITIC, TOPSIS, GIS | Climatic, GHG emission savings, soiling loss | Annual life cycle savings, levelised cost of electricity, net present value, payback period, electricity export revenue | Population density | Energy exported to grid, system losses, temperature losses | Riyadh ranks first (72% performance score). Jeddah follows with 65%. Al Ahsa, Dammam, and Abha score below 50 The findings help identify optimal locations for solar power plants and guide grid expansion strategies. | The presented model could be integrated with machine learning techniques, such as neural networks or decision trees, to improve its accuracy and predictive power. |
| (Watson and Hudson 2015) |  | Wind farm, solar farm | Assessing central Southern England for wind and solar energy suitability | GIS, MCDM, AHP | Distance from wildlife designations, | Distance from transport links, distance from network connection | Distance from historical areas, residential areas | Wind speed, solar radiation | The region is less suitable for wind energy generation than for solar developments. | The presented method can improve investigations into the impact of renewable energy facilities by identifying suitable areas for future developments based on the environmental considerations |
| (Ibrahim, Hamid et al. 2021) |  | Solar power plant | Aims to assess suitable locations for solar power plant installation in Erbil governorate | GIS, Boolean-AHP model | Faults, natura reserves, rivers, slope, elevation | Transmission lines, roads, airport and military area, urban areas, villages, oil and gas |  |  | Final model categorized areas into four suitability levels, identifying 369 candidate sites, with approximately 85% located in the southern and central regions of the study area. | Lacks social and technical factors |
| (Giamalaki and Tsoutsos 2019) |  | Solar power installations | Aims to develop and test a methodology for identifying and prioritizing suitable locations for solar power installations | GIS, AHP | Slope, elevation, land cover |  |  | Distance from the road network, distance from the high voltage lines, slope directions | The methodology was applied to the Regional Unit of Rethymno, identifying high-priority areas with a potential maximum power output of 530 MWp for photovoltaics and 30 MW for Concentrated Solar Power systems. | Lacks socio-economical factors |
| (Wang, Dang et al. 2022) |  | Solar PV plants | Aims to determine suitable locations for solar power installation in Vietnam | DEA, G-AHP, G-TOPSIS | Relative humidity, precipitation, cloudiness, sunshine hours, irradiation, elevation |  |  | Air temperature, wind speed, | The integrated approach enables a thorough evaluation of solar site suitability, aiding cost-effective and resource-efficient decision-making. The study emphasizes the need for government policies and support mechanisms to ensure sustainable solar energy development | Doesn’t include socio-economic factors |
| (Melek, Gökmener et al. 2024) |  | Floating FPV systems | Aims to develop a robust framework for selecting suitable sites for floating photovoltaic systems | Fuzzy Einstein-based Logarithmic Additive Estimation of Weight Coefficients (LAAW) | Orientation of panels, distance to protected areas, distance to seismic activity, ecological impact on marine life, | Distance to main roads, substations, economic performance indicatiors, levelized cost of energy | Employemtn opportunities, policy support, public acceptance, availability of skilled manpower | Capacity factor, corrosion of structural elements, electricity demand, safety of PV plant | The most important criteria are found as “Initial investment” and “Operation and maintenance cost” | This study can be extended with fuzzy Aczel-Alsina functions to provide non-linear flexible processing of uncertain information |
| (Kahatapitiya, Jayasooriya et al. 2023) |  | Solar farms | Aims to develop a GIS-based weighted overlay model to identify suitable locations for wind and solar farm development in Sri Lanka | GIS based weighted overlay model | Elevation, global horizontal irradiation, forest reservations, wildlife reservations, | Land use patterns | Housing density, urban centres, distance to roads, archeology sites | Wind direction, wind speed, distance from aviation ports, transmission lines | The model identified 141 km² as highly suitable for wind farms and 99 km² for solar farms. The most suitable provinces for solar energy were Northern provinces | To concentrate more on economic factors |
| (Abdel-Basset, Gamal et al. 2021) |  | Photovoltaic farms | Aims to identify and prioritize suitable locations for solar photovoltaic farms | Delphi method, DEMATEL, VIKOR | Noise impact, visual impact, land availability, transmission grid accessibility, pollutants and emissions, environmental risk | Infrastructural cost, operation and maintennace, economic risk | Social acceptability, demand for electricity, distance from residential region, government policies, | Solar radiation data, trained manpower, climatic coditions, technology maturity, power generations | The Benban solar PV farm is identified as the most sustainable location, followed by Darb Al Arbaeen. | Lacks geographic information within factors |
| (Mensour, El Ghazzani et al. 2019) |  | Solar farms | Aims to identify and evaluate suitable locations for large-scale photovoltaic projects in southern Morocco | GIS, AHP | Solar irradiation, Land use, distance to urban areas, waterway, dams, food sensitive areas | Roads, flight security, distance to transmission lines, slope |  |  | 24.08% of the studied area is suitable for PV farm development | Lacks socio-economic factors |
| (Rios and Duarte 2021) |  | Photovoltaic power plants | Aims to identify suitable sites for large-scale photovoltaic projects in Peru | MCDA, GIS, AHP | Solar irradiance, slope, land use |  | Proximity to urban areas, to roads | Slope orientation, proximity to transmission lines | The analysis excluded 69.52% of the country, with 22,257 km² classified as highly or very adequate for PV projects | Lacks economic factors |
| (Sari and Rusen 2025) |  | Solar power plants | Aims to evaluate solar energy potential by analyzing the interactions between 16 key criteria and 1,311 existing solar power plants | MaxEnt, Logistic regression, GIS | Radiation, topographic criteria, distance tor roads, powerlines |  |  | Climatic criteria | Key influencing factors included proximity to highways, power transmission lines, land use, and temperature-related criteria | Lacks socio-economic factors |
| (Merrouni, Elalaoui et al. 2018) |  | PV plants | Aims to evaluate the suitability of Eastern Morocco for large-scale photovoltaic farm development | GIS, AHP | Climate, orography, distance from water, dams, groundwater |  | Distance from residential areas, from railway and roads, | Distance from eletricity grid | 19% of the region is highly suitable for PV farms, while 15% is unsuitable. | Lacks economic fators |
| (Elboshy, Alwetaishi et al. 2022) |  | PV solar farms | Aims to identify the most suitable sites for photovoltaic farms in Egypt | MCDM, AHP, GIS | Elevation, slope, soil texture, climatography, meteorology |  |  | Distance from transmission lines, urban areas, major roads | Study developed a suitability map that can guide future renewable energy projects in Egypt | Lacks socio-economic factors |
| (Nagendra, Narayanamurthy et al. 2020) |  | Solar photovoltaic sites | Aims to demonstrate how open innovation using satellite imagery can help the independent assessment of solar photovoltaic sites, with a case study of Bangalore, India | AHP | Land use and cover, height, slope, aspect, road network, water resources |  |  | Solar radiation, temperature, wind direction, wind speed, | Study highlights how satellite imagery-based big data analytics can build investor confidence in the solar industry | Lacks socio-economic fators |
| (Spyridonidou, Loukogeorgaki et al. 2022) |  | PV energy projects | Aims to classify and prioritize municipalities for PV energy projects and identify suitable sites for PV installation in Portugal | GIS, ENTROPY, AHP, TOPSIS | Geographic boundaries, global horizontal irradiance, distance from protected areas, bird areas, average air temperature, |  | Distance from residental and urban areas, from road network, railway network, military zones, agricultural land, religious sites, touristic zones | Distance from electricity grid, | Study identifies several suitable municipalities for PV deployment, with the Municipality of Mértola found to be the most suitable, and concludes by formulating a PV energy roadmap for Portugal | Lacks economic factors |
| (Al Garni and Awasthi 2017) |  | Solar PV power plants | Aims to evaluate and select the best locations for utility-scale solar PV projects | GIS, MCDM | Land use, agrological capacity, climatic, orography, distance to wildlife designations | Land costs, consruction cost | Distance to urban areas, population density, | Distance to substations, to main roads, to power lines, | 16% of Saudi Arabia's land is suitable for utility-scale PV projects, with the most promising areas in the north and northwest | Tackling hybrid systems including more than one RES, such as solar PV-wind and solar-biomass could lead to cost effective and technically feasible RES projects |
| (Munkhbat and Choi 2021) |  | Solar power systems | Aims to identify suitable sites for large-scale solar PV power plant installations in Mongolia | GIS, AHP | Global horizontal irradiation, elevation, slope, aspect |  |  | Temperature, distance from main road, from major power line | 3.27% of the land is highly suitable, mainly in southern and central Mongolia, with the central region prioritized due to its higher electricity demand | Lacks socio-economic factors |
| (Asakereh, Soleymani et al. 2017) |  | Solar photovoltaic farms | Aims to prioritize land in Khuzestan province, Iran, for solar photovoltaic farm installation | AHP, GIS | Slope, waterbodies, agriculture land, protected natural areas, environemntal protection, |  | Residential areas | Roads and railroad network | Findings highlight Khuzestan’s high potential for solar electricity generation, particularly in desert and semi-desert areas, where solar farms could also help combat desertification and reduce overall energy costs. | Lacks economic factors |
| (Li, Dong et al. 2024) |  | Photovoltaic farms | Aims to develop a new evaluation system for solar PV site selection that minimizes subjective decision-making and improves multi-factor consistency | Combination of Geodetector and the Interaction Linear Model | Land use cover, annual solar radiation, annual average temperature, annual rainfall, slope, aspect, |  |  | Electricity network, transportation network, installed capacity of real PV farms | 13% of China’s land as suitable for solar PV development, with Xinjiang being the most optimal region, offering a novel approach to improving renewable energy site selection globally | Lacks socio-economic factors |
| (Yankiv-Vitkovska, Peresunko et al. 2020) |  | Solar power plants | Aims to develop a method for identifying optimal locations for solar power plants near large cities | AHP, ANP, QGIS | Abundant solar irradiation, slope and aspect, land cover, protected areas, shoreline, high altitude areas |  | Proximity to populated area, Proximity to enterprises, Proximity to multi-storey houses, Proximity to residential areas | Transmission lines adjusted to capacity located nearby, Proximity to road network, Average air temperature in July | The method was successfully applied in Zaporizhia, Ukraine, demonstrating its adaptability for other regions and its usefulness for investors in preliminary site selection for solar energy projects | Research lacks economic factors |
| (Kannan, Moazzeni et al. 2021) |  | Solar sites | Aims to identify sustainable locations for solar site development in eastern Iran | BWM, GRA, VIKOR, Monte Carlo simulation-based (MCSB) sensitivity analysis | Protected areas, ecosystem destruction, Distance to catchment basins | Construction cost, maintenance cost, nitial investment, investment risk, economic risk, time delay risk | Job creation, social acceptance, distance to farmlands | Distance to substation, and availability, solar radiation intensity, wind intensity | The study identified Birjand, Sarbisheh, and Khezri as the most suitable locations | Adding a new set of criteria and sub criteria to the proposed model and performing sensitivity analysis of each criterion in final results |
| (Sreenath, Sudhakar et al. 2020) |  | Solar power plant | To develops a site selection procedure for airport-based solar PV systems and identify suitable locations at Senai International Airport, Malaysia | Glare, ForgeSolar | Solar resource, weather aspects, vegetations and ecological aspects, landscape aspects |  |  | Shading and soiling aspects, electric infra aspects, | Out of 11 selected sites, four required design modifications due to glare duration. The total solar PV potential was estimated at 12.50 MW, with a theoretical energy generation of 16,745 MWh | Doesn’t include socio-economic aspects |
| (Badi, Pamucar et al. 2021) |  | Solar power plants | Aims to select optimal locations for solar power plants (SPPs) in Misrata District, Libya | GIS, SWRA,DEMATEL | Land use, slope, altitude, aspect, |  | Distance from residential areas, | Solar iradiation, distance from transport links, distance from electricity0-transmission network links | 1,667 km² in Misrata as highly suitable for SPP development | Lacks economic factors |
| (Tercan, Eymen et al. 2021) |  | Photovoltaic solar farms | Aims to support the expansion and spatial planning of photovoltaic (PV) solar farms by identifying optimal locations | AHP, WLC, IDW, GIS | GHI, aspect, slope, elevation, land use/cover, distance from protected areas, distance from rivers and lakes | Distance from land and railway, transportation network |  |  | 3.70% of the area is highly suitable, while 40.01% is moderately suitable for PV farms, with the western region identified as the most optimal | Lacks social and technical factors |
| (Hassaan, Hassan et al. 2021) |  | Photovoltaic power plants | Aims to identify suitable locations for photovoltaic (PV) power plants in Kuwait | Multi criteria analysis, GIS | Solar irradiation, physical suitability, land availability | Cost effectiveness |  |  | Study identified potential sites for solar energy plants in Kuwait and evaluated the potential electricity generation capacity of these locations | Doesn’t include social and technical factors |
| (Günen 2021) |  | PV power plants | Aims to explore optimal site selection for large-scale solar PV power plants in Kayseri, Turkey | MCDM , AHP, GIS | Global horizontal irradiation, aspect, slope, land use/cover, distance from bird migration routes, distance from faults, from rivers and water surfaces, | Distance from road and railway networks |  | Distance from power lines and transformer centres, | The most suitable districts for solar PV installation were identified as Sarız, Tomarza, and Incesu | Lacks social factors |
| (Suuronen, Lensu et al. 2017) |  | Photovoltaic power plants | Aims to optimize the location of photovoltaic solar power plants in the Atacama Desert, Chile | MCDA, AHP, OWA, GIS | Physical factors, biomass, vegetation type rarity, land use |  | Distance from cities, landscape |  | The most suitable sites were near roads and power lines in the Andes, and the MCDA-GIS method was found to be effective, with suitability increasing when certain constraints like distance to roads and power lines were removed | Lacks economic and technical factors |
| (Agyekum, Amjad et al. 2021) |  | Photovoltaic power plants | Aims to select optimum site for the development of solar power plants are identified in Ghana | AHP, GIS | Solar irradiance, protected areas, slope, Land use |  | Distance to urban settlements | Proximity to transportation, to transmission lines | Wa in the Upper West region was identified as the top priority location, followed by two other macro clusters | Lacks economic factors |
| (Habib, Suliman et al. 2020) |  | Photovoltaic power plant | Aims to identify the most suitable locations for establishing photovoltaic (PV) power plants in Egypt | GIS, RS, AHP | LULC, land capability | Solar radiation, slope, distance for build-up area, major road and railway, power transmission lines |  | Shadow, aspect, weather condition | 24.9% of the study area (261.1747 km²) is highly suitable for deploying PV power plants, with solar radiation values ranging from 4.7 to 5.9 kWh/m²/day | Lacks social factors |
| (Mokarram, Mokarram et al. 2020) |  | Solar photovoltaic farms | Aims to develop a novel framework for determining the optimal location for constructing solar photovoltaic (PV) farms | AHP, DS, GIS | Number of dusty days, cloudy days, land use, land elevation, land slope |  | Distance to residential area, to major road | Solar radiation intensity, relative humidity, temperature, distance to PTL | 32% of the study area is highly suitable according to the fuzzy_AHP method, while the fuzzy_DS method identifies 18.56%, 16.70%, and 16.32% of the area at different confidence levels | Doesn’t include economic factors |
| (Lindberg, Birging et al. 2021) |  | PV power park | Aims to identify suitable land for utility-scale solar photovoltaic (PV) parks and assess the hosting capacity of the local grid in Sweden | Multi-criteria analysis with a Boolean approach, GIS | Water, natural reserves, agricultural land | Roads/railway | Buildings, nationally designated areas, | Wind power, | 3.7% of the study area is suitable for 1 MWp PV parks, with potential further reduced by distance thresholds to substations. Larger PV parks could be located near urban areas, but suitable land is limited | More economic factors can be included |
| (Shorabeh, Firozjaei et al. 2019) |  | Solar power plant | Aims to identify potential locations for solar power plants in four provinces of Iran | GIS-MCDA, OWA | Slope, solar radiation, rainfall, dust, sunshine, distance from fault |  | Distance from city, | NDVI, LST, distance from road | Study finds that arid climate provinces like Yazd are more suitable for solar power plants than wet climate provinces, and the criteria "slope" and "road network" have the greatest influence on suitability | Doesn’t include economic factors |
| (Rekik and El Alimi 2024) |  | Solar PV farms | Aims to determine the most suitable locations for large-scale solar PV power plants in Tunisia | GIS, MCDM, FAHP | Climatography, topography, prox. To water | Prox, to road | Prox. to urban | Prox. To grid, | 17.6% of the area as suitable for solar PV development, with the most suitable sites covering 5251 km² in the central, southwestern, southeastern, and eastern coastal regions of Tunisia | More social and economic factors can be included |
| (Sun, Jiang et al. 2021) |  | PV and CSP plants | Aims to evaluate suitable locations and technical potentials for large-scale solar photovoltaic (PV) and concentrated solar power (CSP) plants | GIS, AHP | Solar irradiation, average temperature, slope, water availability | Proximity to road and railway network, proximity to load and demand area |  | Proximity to power grid | The middle area of Ningxia, China, is identified as the most suitable for solar power plants, with PV being more appropriate than CSP due to water resource limitations, and the technical potential for PV and CSP systems estimated at 443 TWh/year and 308 TWh/year | Doesn’t include social factors |
| (Hashemizadeh, Ju et al. 2020) |  | Photovoltaic power plants | Aims to introduce an evaluation model for determining the optimal location for a photovoltaic project | BWM, GIS | Climatic, orography, waterway | Land cost, construction cost | Distance to urban areas, to main roads | Distance to power lines | 27.4% of Beijing’s region is suitable for solar projects, with 61.96% of the suitable areas classified as "most suitable" and 27.14% as "very suitable." | Social and cultural policies can be included in criteria |
| (Kocabaldır and Yücel 2023) |  | Photovoltaic power plants | Aims to select optimal sites for solar photovoltaic power plants in Çanakkale province | GIS, AHP | Slope, aspect, average annual temperature, relative humidity, distance to water resources, fault lines | Distance to highways, | Distance to residential areas | Solar irradiation, distance to transmission lines, substations, | 96% of the study area is restricted, with only 3‰ and 3% of the area classified as most and highly suitable | Considering the hosting capacity of the distribution grid can be considered |
| (Bedewy and Al-Timimy 2023) |  | Solar power plants | Aims to identify optimal locations for solar energy farms in Babylon, Iraq | AHP, GIS |  | Distance from road network, distance from cities centre |  | Solar radiation, Earth's surface inclination, distance from electricity distribution network, residential compounds boundaries, agricultural lands | Suitable locations cover 2% to 37% of the study area, ranging from 10 km² to 155 km², with a total of 422 km² proposed for solar energy projects | Doesn’t include social factors |
| (Ruiz, Sunarso et al. 2020) |  | Solar power plants | Aims to develop site-suitability assessment tool for solar power plants | AHP, MCDA, GIS | GHI, temperature, relative humidity, digital elevation model, slope, aspect, | Road infrastructure, grid infrastructure | Settlements |  | While WKP has high Global Horizontal Irradiation (GHI) value across its area, accounting for protected areas reduces the available land for solar deployment to 34% | Can include more social and technical factors |
| (Sun, Zhu et al. 2023) |  | Photovoltaic power plants | Aims to investigate optimal site selection for large-scale solar photovoltaic (PV) power plants | Machine learning models, SHAP, GIS | Land cover, slope, aspect, elevation, distamce to water resources, surface concentrationtemperature, carbon emission, solar irradiation, sunshine duration | Land cost, transportation convenience, gross GDP, electricity consumption, electricity support | Distance to residential area, distance to power grid,population density, |  | The most suitable regions for solar PV development covered 4.6% of China’s land area | More technical factors can be included for using method for other regions |
| (Halder, Banik et al. 2022) |  | Solar power plant | Aims to evaluate optimal site selection for solar power plants in Kolkata and surrounding areas | AHP, MCDA, WOA, GIS | Land use, slope, aspect, altitude, soil type, solar radiation, water body, protected area |  | Residential area, highway | LST | 10.69% of the study area was classified as "very highly suitable" for solar power plants. 32.43% was categorized as "highly suitable." 5.62% was deemed "not suitable," | Can integrate more economic factors |
| (Settou, Settou et al. 2022) |  | Photovoltaic plant | Aims to develop an interdisciplinary framework for optimal mapping and integration of solar photovoltaic (PV) distributed generation (DG) systems | GIS, MCDM, BFS, VSI | Prox. to vegetation areas, slope, distance to lakes | Pro. To power line, prox. to roads, | Prox. to urban area, |  | 78% of the study area is suitable for PV farm installation | Lacks more social and technical factors |
| (Fard, Moradian et al. 2022) |  | Ground-mounted photovoltaic power station | To evaluate the optimal location for a solar power plant in Guilan province | FBWM, GIS | Land use, distance from protected areas, from surface water, | Distance from main roads, Distance from faults, slope, | Distance from urban and rural areas, from historical areas, | Distance from power lines, wind, temperature, GHI | 0.74% of Guilan province is classified as "most suitable" for solar power plants | Further analysis of financial factors within economic criteria |
| (Shorabeh, Samany et al. 2022) |  | Solar power plants | To identify optimal locations for solar power plants | MCDM, GIS | Solar radiation, slope, NDVI |  | Fault, road, cities | Wind speed, air temperature, humidity, zenith angle, azimuth angle, pressure, total ozone, lifted index, K-indexwater vapor, LST | The decision tree method demonstrated higher accuracy, making it the preferred technique for identifying optimal solar power plant locations | Could include more socio-economic actors |
| (Ouchani, Jbaihi et al. 2021) |  | Large-scale photovoltaic installations | To identify suitable sites for PV power plants in the Marrakesh-Safi region, Morocco | MCDM, GIS | Solar resource, slope, distance from water resources, land use, aspect, elevation, elevation, agrological capacity, cloudy days, dust days, fault lines, nature gas line, plot area | Distance from roads | Distance from urban areas, | Distance form grid, temperature, humidity, electricity from PV system | 74% of the study area was suitable for PV power plants | Could include more socio-economic factors |
| (Minaei, Minaei et al. 2021) |  | Photovoltaic systems | Evaluates the feasibility of solar photovoltaic (PV) technology for electrifying remote settlements in protected areas of Khorasan-e-Razavi, Iran | BWM, GIS | Aspect, prox. to rivers, prox. to faults, solar radiation, solar duration, sunny hours, |  | Prox. to roads, | Humidity, precipitation, temperature | 2% of the region is very highly suitable for solar PV, Rooftop PV systems could meet the residential electricity demand | Doesn’t include economic factors |
| (Heidary Dahooie, Husseinzadeh Kashan et al. 2022) |  | Solar power plants | Aims to develop framework to select optimal solar power plant (SPP) locations in Iran | MCDM, MULTIMOORA, GIS | Average temperature, distance from wildlife and protected areas, distance from agricultural lands, slope, average elevation, orientation, land use, distance from water resource | Distance from transportation network, | Distance from urban area, | Distance from power line, | The proposed framework ensures a more accurate and robust site selection. Results align with previous studies, validating the effectiveness of the method | More social factors can be included |
| (Jbaihi, Ouchani et al. 2024) |  | Larg-scale CSP | Aims to evaluate Morocco’s technical potential for Concentrating Solar Power (CSP) and compares site suitability assessment methods | AHP, FAHP, GIS | DNI, slope, distance from water ways, from dams, | Distance from road and railway network, from airports, | Distance from cities and urban areas | Distance from electricity grid | Climate was the most influential facto, while location had the least influence | More social and technical factors can be included |
| (Rediske, Siluk et al. 2020) |  | Large photovoltaic farms | Aims to identify optimal locations for large-scale photovoltaic (PV) power plants | AHP, TOPSIS, GIS | Watercourses and streams, protected lands, waterways, Areas with inclination | Road networks, railway | Urban lands |  | 67.23% of identified areas were inadequate for PV installation | Could include more social and technical factors |